Glass cylinder safety luminaire with explosion protection for gas and dust Ex zones. 

Prepared for two cable entries for feed-through wiring. By attaching a pictogram, can also be used as an emergency exit sign luminaire. 

By means of mounting adapter for wall/ceiling mounting or suspension by means of eye bolt. 

Labelling according to 2014/34/EC (ATEX)

Zone 1 / zone 21 gas and dust

- Ex d IIB T6 Gb / Ex d IIB T6 Gb
- Ex t IIIC T85ºC Db

Manufactured in accordance with
-EN 60079-0:2018, EN 60079-1:2014, EN 60079-31:2014

-Certified by INERIS 20ATEX0005X

Material: Aluminum die-cast,Glass

Color: Gelb

Dimensions: 1360 mm x 157 mm x 197 mm

Mounting method: Universal

Protection class: 1

Protection rating (IP): IP 66

Impact restistence rate IK: IK 7

Allowed temperature DS: -20 °C to 55 °C °C

Allowed temperature BS: -20 °C to 55 °C °C

Pictogram: Nein

Power maintained mode: 40 W W

Luminous Flux Emergency Operation: 4600 lm lm

Input voltage AC: 230 V V

Connection terminals: 2.5 mm² mm

Battery:

Article number: XGXU009

Accessories:

Article number: X-MB001, Mounting bracket (pair) wall / ceiling for XI /  XM / XG

Article number: X-KE001, Metal cable gland with strain relief for XI, XM,  XG

Article number: X4N, Pictogram set VD 14m MTO, MTU, MTL, MTR including internal holder for XI / XM / XG

Article number: X-KO002, Chain eyelets (pair) for XI / XM / XG

Brand: RP-Technik GmbH