Polycarbonate cylinder safety luminaire with explosion protection for gas and dust Ex zones. 

Prepared for two cable entries for feed-through wiring. By attaching a pictogram, can also be used as an emergency exit sign luminaire. 

Suitable for wall or ceiling mounting using mounting adapters or suspension via chain eyelet.

Labelling according to 2014/34/EC (ATEX)

Zone 1 / zone 21 gas and dust

- Ex d IIB T6 Gb / Ex d IIB T6 Gb
- Ex t IIIC T85ºC Db

Manufactured in accordance with
-EN 60079-0:2018, EN 60079-1:2014, EN 60079-31:2014

-Certified by INERIS 20ATEX0005X

Monitoring:

Version with automatic test system in accordance with DIN EN 62034 type S including SelfControl test button:

- Test results with fault analysis (illuminant, charge and battery circuit) and status displays (operation, function test, continuous function test) using 3 colour LEDs

- Automatic function test (weekly)

- Automatic continuous function test (every 12 months)

- Automatic charge monitoring

- Deep discharge protection with restart inhibit

- Idling and short-circuit shutdown of inverter

Material: Aluminum die-cast,Polycarbonate

Color: RAL 5010

Dimensions: 485 mm x 157 mm x 197 mm

Mounting method: Universal

Protection class: 1

Protection rating (IP): IP 66

Impact restistence rate IK: IK 5

Allowed temperature DS: 0 °C to 55 °C °C

Allowed temperature BS: 0 °C to 55 °C °C

Pictogram: Nein

Power maintained mode: 24 W W

Power non-maintained mode: 3 W W

Luminous Flux Emergency Operation: 750 lm lm

Input voltage AC: 230 V V

Connection terminals: 2.5 mm² mm

Battery:

Article number: XPIU011SC

Accessories:

Article number: X-KE001, Metal cable gland with strain relief for XI, XM,  XG

Article number: X-MB001, Mounting bracket (pair) wall / ceiling for XI /  XM / XG

Article number: X-KO001, Chain eyelets (pair) for XI / XM / XG

Article number: X4N, Pictogram set VD 14m MTO, MTU, MTL, MTR including internal holder for XI / XM / XG

Brand: RP-Technik GmbH