Combined LED general and emergency luminaire for the illumination of escape and rescue routes according to DIN EN 60598-1, DIN EN 60598-2-22 and DIN EN 1838. 
The robust LED wall and ceiling luminaire made of glass-fibre reinforced polyester for damp locations is suitable for illuminating storage areas, multi-storey car parks, etc. In mains operation, the luminaire has an output of up to 7,000 lm. Easy quick mounting by means of metal mounting bracket.

Monitoring:

Version with automatic test system in accordance with DIN EN 62034 type S including SelfControl test button:

- Test results with fault analysis (illuminant, charge and battery circuit) and status displays (operation, function test, continuous function test) using 3 colour LEDs

- Automatic function test (weekly)

- Automatic continuous function test (every 12 months)

- Automatic charge monitoring

- Deep discharge protection with restart inhibit

- Idling and short-circuit shutdown of inverter

Material: Polycarbonate

Color: RAL 7035

Dimensions: 1560 mm x 82 mm x 97 mm

Mounting method: Universal

Protection class: 1

Protection rating (IP): IP 65

Impact restistence rate IK: IK 8

Allowed temperature DS: -5 °C to 40 °C °C

Allowed temperature BS: -5 °C to 40 °C °C

Pictogram: Nein

Power maintained mode: 55 W W

Power non-maintained mode: 0,7 W W

Luminous Flux Emergency Operation: 500 lm lm

Input voltage AC: 230 V V

Connection terminals: 1.5 mm² mm

Battery: LFP3245.WRLED

Article number: LFD15003SC-NW

Accessories:

Brand: RP-Technik GmbH