Die-cast zinc disc light in accordance with DIN EN60598-1, DIN EN 60598-2-22 and DIN EN 1838.

German Design Award 2018 winner: zinc die-cast LED disc light for wall mounting. 

The simple and clear design language of all A-series light variants allows them to be integrated into any type of building. The modular, practical design of the A-series promises easy installation for all variants. The design is perfectly adapted to any type of installation. 

Features:
- Mechanically robust and corrosion-resistant
- Powder-coated surface
- Precise edges and straight lines
- No weld seams or bent edges
- Solvent-free attachment of pictogram
- Disc with polished edges and continuous, invisible pictogram holder
- Mounting housing identical for all A-series variants.

Suitable for permanent or standby switching. Planning reliability thanks to tool-free, variable use of pictograms on site. Pictograms in accordance with DIN EN ISO 7010 and DIN ISO 3864 (left, right, bottom, top) included as standard.

Monitoring:

With integrated monitoring module with 20-digit addressing switch, for operation on a central power supply system. Automatic test system according to DIN EN 62034 type ER, in connection with a system of type MultiControl. Functions must be complied with:

- Integrated individual luminaire monitoring

- Integrated luminaire manager function

- Individual switchability in conjunction with RP ELS - central power supply system

Material: Zinc die-cast

Color: RAL 7016

Dimensions: 55 mm x 193 mm x 120 mm

Mounting method: Wandaufbau

Protection class: 2

Protection rating (IP): IP 40

Impact restistence rate IK: IK ≥ 3

Allowed temperature DS: -5 °C to 40 °C °C

Allowed temperature BS: -5 °C to 40 °C °C

Viewing distance: 15m m

Pictogram: Set

Power maintained mode: 2,6 W W

Power non-maintained mode: 1,1 W W

Input voltage AC: 230 V V

Connection terminals: 2.5 mm² mm

Battery:

Article number: AIW009ML-AZ

Accessories:

Brand: RP-Technik GmbH