Plastic escape sign luminaire for universal mounting. Universal mounting allows the luminaire to be mounted either on the wall or ceiling. The cuboid plastic housing is discreetly reduced to the essentials. The high degree of protection enables use in damp rooms or environments with high levels of soiling.
Display disc light with integrated light distribution for uniform illumination of the pictogram. Suitable for single or double-sided labelling.

Planning reliability is guaranteed thanks to the tool-free, variable use of pictograms on site. A pictogram set in accordance with DIN ISO 7010 (right, left, up and down arrows) is included as standard.

Monitoring:

Version with automatic test system in accordance with DIN EN 62034 type S including SelfControl test button:

- Test results with fault analysis (illuminant, charge and battery circuit) and status displays (operation, function test, continuous function test) using 3 colour LEDs

- Automatic function test (weekly)

- Automatic continuous function test (every 12 months)

- Automatic charge monitoring

- Deep discharge protection with restart inhibit

- Idling and short-circuit shutdown of inverter

Material: Polycarbonate

Color: RAL 9003

Dimensions: 326 mm x 50 mm x 204 mm

Mounting method: Universal

Protection class: 2

Protection rating (IP): IP 65

Impact restistence rate IK: IK 7

Allowed temperature DS: -5 °C to 40 °C °C

Allowed temperature BS: -5 °C to 40 °C °C

Viewing distance: 22m m

Pictogram: Set

Power maintained mode: 3,3 W W

Power non-maintained mode: 0,9 W W

Luminous Flux Emergency Operation: 180 lm lm

Input voltage AC: 230 V V

Connection terminals: 2,5 mm² mm

Battery: LFPN3212.01-SET

Article number: KAMU401SC

Accessories:

Article number: AWX001, Wall bracket, white convenient for RX, KB, KX, KW, LX, GX, KE

Article number: 2PW-EB, Double - Pendulum white 500mm  pre-wired with 4-wires

Article number: 2DW-EB

Brand: RP-Technik GmbH