Emergency sign luminaire in accordance with DIN EN 60598-1, DIN EN 60598-2-22 and DIN EN 1838. 
  
Slim, elegant, convex plastic luminaire inclined towards the viewer, tapering symmetrically downwards. For universal mounting (wall/ceiling). 
Suitable for continuous or standby operation. Planning reliability thanks to tool-free, variable use of pictograms on site. 
The pictograms comply with DIN ISO 7010 (2x man in door and two arrows) and are  inserted internally and are solvent-free.

Monitoring:

With integrated monitoring module, for operation on a 24V power supply system. Automatic test system in accordance with DIN EN 62034 type ER, in conjunction with a CoreCompact system. Functions must be complied with:

- Integrated individual luminaire monitoring

- Automatic addressing via configuration wizard

- Luminaires can be individually programmed and dimmed with RP's central power supply system

- Luminaire update via the system

Material: Polycarbonate

Color: RAL 9003

Dimensions: 212 mm x 31 mm x 141 mm

Mounting method: Universal

Protection class: 3

Protection rating (IP): IP 40

Impact restistence rate IK: IK 5

Allowed temperature DS: -30 °C to 40 °C °C

Allowed temperature BS: -30 °C to 40 °C °C

Viewing distance: 20m m

Pictogram: Set

Power maintained mode: 2,00 W W

Power non-maintained mode: 0,4 W W

Battery:

Article number: KFMU509CC

Accessories:

Brand: RP-Technik GmbH