LED emergency sign light in accordance with DIN EN 60598-1, DIN EN 60598-2-22, DIN EN 7010 and DIN EN 1838

Plastic discs – emergency sign light for surface mounting on ceilings and for marking emergency and escape routes. Levelling screws are integrated to facilitate installation and compensate for uneven surfaces. Planning reliability is ensured by the tool-free, variable use of pictograms on site. The pictograms are inserted without the use of solvents. The pictogram set (arrows up, down, right and left) is included as standard.

Monitoring:

Version with automatic test system in accordance with DIN EN 62034 type S including SelfControl test button:

- Test results with fault analysis (illuminant, charge and battery circuit) and status displays (operation, function test, continuous function test) using 3 colour LEDs

- Automatic function test (weekly)

- Automatic continuous function test (every 12 months)

- Automatic charge monitoring

- Deep discharge protection with restart inhibit

- Idling and short-circuit shutdown of inverter

Material: Polycarbonate

Color: RAL 9003

Dimensions: 80.9 mm x 417.5 mm x 245.5 mm

Mounting method: Deckenaufbau

Protection class: 2

Protection rating (IP): IP 40

Impact restistence rate IK: IK 5

Allowed temperature DS: -5 °C to 40 °C °C

Allowed temperature BS: -5 °C to 40 °C °C

Viewing distance: 40m m

Pictogram: Einzeln n.A.

Power maintained mode: 3,3 W W

Power non-maintained mode: 0,9 W W

Input voltage AC: 250 V V

Battery: LFPN3212.01-SET

Article number: GABD403SC

Accessories:

Brand: RP-Technik GmbH