LED safety luminaire for illuminating escape and rescue routes according to DIN EN 60598-1, DIN EN 60598-2-22 and DIN EN 1838 

Robust swivelling LED spotlight for wall and ceiling mounting made of die-cast zinc.

The open beam characteristic ensures uniform light distribution. 
Possible applications are:
- Open areas, escape route intersections
- High rooms, warehouses

Monitoring:

With integrated monitoring module with 20-digit addressing switch, for operation on a central power supply system. Automatic test system according to DIN EN 62034 type ER, in connection with a system of type MultiControl. Functions must be complied with:

- Integrated individual luminaire monitoring

- Integrated luminaire manager function

- Individual switchability in conjunction with RP ELS - central power supply system

Material: Zinc die-cast

Color: RAL 9003

Dimensions: 247 mm x 187 mm x 37 mm

Mounting method: Universal

Protection class: 1

Protection rating (IP): IP 66

Impact restistence rate IK: IK 8

Allowed temperature DS: -25 °C to 40 °C °C

Allowed temperature BS: -25 °C to 40 °C °C

Pictogram: Nein

Power maintained mode: 51,5 W W

Power non-maintained mode: 1,5 W W

Luminous Flux Emergency Operation: 8163 lm lm

Input voltage AC: 230 V V

Battery:

Article number: FLU50ML-WS

Accessories:

Article number: MU05-BOX, monitoring module, CBS + ML / 230V AC/DC 4-200 VA  with sep. housing IP54

Brand: RP-Technik GmbH