Combined LED general and emergency luminaire for illuminating escape and rescue routes in accordance with DIN EN 60598-1, DIN EN 60598-2-22 and DIN EN 1838.
The robust LED wall and ceiling light made of glass fibre reinforced polyester for damp room applications is suitable for illuminating storage areas, multi-storey car parks etc.
Quick and easy installation is ensured by means of metal mounting brackets.

Monitoring:

With integrated monitoring module with 20-digit addressing switch, for operation on a central power supply system. Automatic test system according to DIN EN 62034 type ER, in connection with a system of type MultiControl. Functions must be complied with:

- Integrated individual luminaire monitoring

- Integrated luminaire manager function

- Individual switchability in conjunction with RP ELS - central power supply system

Material: Polycarbonate

Color: RAL 7035

Dimensions: 1560 mm x 82 mm x 97 mm

Mounting method: Wandaufbau,Deckenaufbau

Protection class: 1

Protection rating (IP): IP 66

Impact restistence rate IK: IK 8

Allowed temperature DS: -20 °C to 40 °C °C

Allowed temperature BS: -20 °C to 40 °C °C

Pictogram: Nein

Power maintained mode: 55 W W

Power non-maintained mode: 1,5 W W

Luminous Flux Emergency Operation: 7700 lm lm

Input voltage AC: 230 V V

Connection terminals: 2.5 mm² mm

Battery:

Article number: LFU15009ML-CCT

Accessories:

Brand: RP-Technik GmbH