Combined LED general and emergency luminaire for illuminating escape and rescue routes in accordance with DIN EN 60598-1, DIN EN 60598-2-22 and DIN EN 1838.
The robust LED wall and ceiling light made of glass fibre reinforced polyester for damp room applications is suitable for illuminating storage areas, multi-storey car parks etc.
Quick and easy installation is ensured by means of metal mounting brackets.

Monitoring:

Version with automatic test system in accordance with DIN EN 62034 type S including SelfControl test button:

- Test results with fault analysis (illuminant, charge and battery circuit) and status displays (operation, function test, continuous function test) using 3 colour LEDs

- Automatic function test (weekly)

- Automatic continuous function test (every 12 months)

- Automatic charge monitoring

- Deep discharge protection with restart inhibit

- Idling and short-circuit shutdown of inverter

Material: Polycarbonate

Color: RAL 7035

Dimensions: 1560 mm x 82 mm x 97 mm

Mounting method: Wandaufbau,Deckenaufbau

Protection class: 1

Protection rating (IP): IP 66

Impact restistence rate IK: IK 8

Allowed temperature DS: -5 °C to 40 °C °C

Allowed temperature BS: -5 °C to 40 °C °C

Pictogram: Nein

Power maintained mode: 54 W W

Power non-maintained mode: 0,7 W W

Luminous Flux Emergency Operation: 590 lm lm

Input voltage AC: 240 V V

Connection terminals: 2,5 mm² mm

Battery: LFPN3260.WRLEN, {{Produkt - BatteryPerformance - BatteryTechnology (P:17:110)}} Battery

Article number: LFU15401SC-CCT

Accessories:

Brand: RP-Technik GmbH