Kombinierte LED-Allgemein- und Sicherheitsleuchte zur Ausleuchtung der Flucht- und Rettungswege nach DIN EN 60598-1, DIN EN 60598-2-22 und DIN EN 1838. 
Die robuste LED-Wand- und Deckenleuchte aus glasfaserverstärktem Polyester für Feuchtraumanwendungen eignet sich zur Beleuchtung von Lagerflächen, Parkhäusern etc. 
Eine leichte Schnellmontage ist mittels Metall-Befestigungsbügeln gegeben.

Monitoring:

Version with automatic test system in accordance with DIN EN 62034 type S including SelfControl test button:

- Test results with fault analysis (illuminant, charge and battery circuit) and status displays (operation, function test, continuous function test) using 3 colour LEDs

- Automatic function test (weekly)

- Automatic continuous function test (every 12 months)

- Automatic charge monitoring

- Deep discharge protection with restart inhibit

- Idling and short-circuit shutdown of inverter

Material: Polycarbonate

Color: RAL 7035

Dimensions: 1220 mm x 82 mm x 97 mm

Mounting method: Wandaufbau,Deckenaufbau

Protection class: 1

Protection rating (IP): IP 66

Impact restistence rate IK: IK 8

Allowed temperature DS: -5 °C to 40 °C °C

Allowed temperature BS: -5 °C to 40 °C °C

Pictogram: Nein

Power maintained mode: 36 W W

Power non-maintained mode: 0,7 W W

Luminous Flux Emergency Operation: 530 lm lm

Input voltage AC: 240 V V

Connection terminals: 2,5 mm² mm

Battery: LFPN3260.WRLEN, {{Produkt - BatteryPerformance - BatteryTechnology (P:17:110)}} Battery

Article number: LFU12403SC-CCT-D

Accessories:

Brand: RP-Technik GmbH