Exit sign / escape route light according to DIN EN 60598-1, DIN EN 60598-2-22 and DIN EN 1838

Classic bar-shaped polycarbontate luminaire for ceiling mounting. Straight lines as well as the robust construction allow versatile use.
Suitable for permanent or standby switching. Planning security through tool-free, variable use of the pictograms on site. Pictograms according to DIN EN ISO 7010 and DIN ISO 3864 (left, right, above, below) are included as standard in the scope of delivery.

Monitoring:

Integrated monitoring module for operation with a DALI 2 type monitoring device

- Version with automatic test system according to DIN EN 62034; the type depends on the control unit.

- Automatic test results with fault analysis and status display (operation, function test, endurance test) centrally and at the luminaire via three LEDs.

- Manual or automatic function test configurable by group (test start time freely selectable in the DALI software).

- Configurable endurance test (every 12 months).

- Automatic charging monitoring.

- Deep discharge protection with restart lockout.

Material: Polycarbonate

Color: RAL 9003

Dimensions: 86 mm x 390 mm x 99 mm

Mounting method: Wandaufbau

Protection class: 2

Protection rating (IP): IP 65

Impact restistence rate IK: IK 6

Allowed temperature DS: -5 °C to 35 °C °C

Allowed temperature BS: -5 °C to 35 °C °C

Viewing distance: 16m m

Pictogram: Set

Power maintained mode: 4,7 W W

Power non-maintained mode: 1,0 W W

Luminous Flux Emergency Operation: 335 lm lm

Connection terminals: 2,5 mm² mm

Battery: LFPN3233.01-TT

Article number: KWIW413DL-TT

Accessories:

Article number: BKW, Ball protection (KW) 420x170x170mm RAL9010

Article number: KWIE, KWI - Flush mounting frame for wall and  ceiling installation, white metal

Brand: RP-Technik GmbH