Emergency sign luminaire in accordance with DIN EN 60598-1, DIN EN 60598-2-22 and DIN EN 1838

Plastic LED emergency sign luminaire with convex cover for wall mounting. Suitable for continuous or standby operation. Levelling screws are integrated to compensate for uneven surfaces and facilitate installation. Planning reliability is ensured by the tool-free, variable use of pictograms on site. The pictograms are internal and are inserted without the use of solvents. The pictogram set (MTL, MTR and 2x straight arrow) is included as standard.

Monitoring:

Integrated monitoring module for operation with a DALI 2 type monitoring device

- Version with automatic test system according to DIN EN 62034; the type depends on the control unit.

- Automatic test results with fault analysis and status display (operation, function test, endurance test) centrally and at the luminaire via three LEDs.

- Manual or automatic function test configurable by group (test start time freely selectable in the DALI software).

- Configurable endurance test (every 12 months).

- Automatic charging monitoring.

- Deep discharge protection with restart lockout.

Material: Polycarbonate

Color: RAL 9003

Dimensions: 65 mm x 236 mm x 136 mm

Mounting method: Wandaufbau

Protection class: 2

Protection rating (IP): IP 65

Impact restistence rate IK: IK 6

Allowed temperature DS: -5 °C to 40 °C °C

Allowed temperature BS: -5 °C to 40 °C °C

Viewing distance: 20m m

Pictogram: Set

Power maintained mode: 3,1 W W

Power non-maintained mode: 1,0 W W

Luminous Flux Emergency Operation: 210 lm lm

Battery: LFPN3233.01-TT

Article number: PMW403DL-TT

Accessories:

Article number: PME, Recessed frame ABS white for PMD and PMW  luminaires

Article number: PM-PIKTOSETSONDER.01, Set consists of:  2x Rolli (R&L)  2x Arrow oblique

Brand: RP-Technik GmbH